# Dino v0.1.1 — Instruction Decoder + 7-Segment Display Upgrade

## Objective

Upgrade Dino v0.1.0 by adding SN74LS138N and two 7-segment display drivers.

## Additions

- SN74LS138N 3-to-8 Line Decoder

  - Connected to D0–D2 of the SRAM data bus

  - G1 = HIGH, G2A = G2B = LOW

  - Output lines:

    - Y1 → Register A latch (LOAD_A)

    - Y2 → Register B latch (LOAD_B)

    - Y3 → ALU control (ADD)

    - Y5 → Output latch clock (OUT)

- Two 7-Segment Display Drivers

  - 4-bit BCD inputs from 8-bit ALU output bus

  - D0–D3 → right digit

  - D4–D7 → left digit

## Reserved Values and Constraints

- Instruction Opcodes: 0x00 through 0x07

- Safe Output Range: 0x08 to 0xFF

- Effective Addition Range: 0x08 to 0x5B (decimal 8–91)

## Completion Criteria

- Autonomous execution

- ALU result latched and displayed on 7-segment digits

- Decoder triggers control signals from instruction bytes

- Programs avoid output values < 0x08

## Result

Autonomous operation with limited output range.