# DINO v0.0.1

Wiring and Debug Summary – June 22, 2025

Power & Ground Discipline - Always confirm VCC/GND orientation from the power supply. - Reversed polarity can cause 555 or other ICs to heat up or fail. - Establish local ground nodes early to simplify wiring and decoupling.

555 Timer Bring-Up - LED + resistor off pin 3 for visual test. - Output blink confirmed using 1µF electrolytic cap; positive to timing node, negative to GND. - Capacitance value directly affects blink speed; 68nF is fast, 1µF is slow. - Control voltage pin (pin 5) should be tied to GND via 0.01-0.1µF ceramic cap.

LED Orientation - Anode to current source (e.g., 555 output), cathode to GND. - Reversed LED will not light.

Reset Subsystem - MRC (~CCLR) is active-low; must be pulled high with 10kΩ resistor. - Button connects MRC to GND for manual reset. - Shared pull-up resistor is acceptable across multiple MRC pins.

Program Counter (PC) with 74LS590 - Stage A: - ~CE pulled LOW (enabled at all times). - CPC clocked from 555 timer. - Stage B: - ~CE driven by RCO of Stage A (ripple carry). - CPC also clocked from 555. - CPR tied HIGH for real-time output. - ~OE on both chips tied LOW to enable output. - B counts 256x slower than A. - RCO is a brief pulse, not a latched HIGH.

Debug Findings - Confirmed that both ~CE pulled HIGH disables counting. - Correct chaining requires: - 555 clock -> both CPC - ~CE A -> LOW - ~CE B <- RCO A

Suggested Visual Debug Aids - Add LED on B.Q0 to visualize slower pulse (1 per 512 clocks).

General Practices - Test subsystems incrementally. - Watch pin functions and active-low indicators (~). - Avoid floating inputs; tie with pull-ups or pull-downs. - LED test rig is useful for visual validation of digital outputs.

Next Steps - Complete schematic corrections (~CE wiring). - Validate full 16-bit PC operation. - Proceed to SRAM address decoding and wiring from PC bus.

DINO is alive. Power is disciplined. Timers are stable. Counter chain is operational.