# Dino v0.01 — Build Instructions and Bill of Materials

## Objective

Construct a manual, CPU-free logic sequencer capable of stepping through memory and outputting 8-bit values using LEDs. This stage establishes the address counter, SRAM interface, manual program loading, and basic execution loop.

## Bill of Materials (Dino v0.01)

| Qty | Part | Description |
| --- | --- | --- |
| 2 | SN74LS590N | 8-bit binary counters for 16-bit address |
| 1 | MB81256-10 | 32KB SRAM for program/data memory |
| 1 | 555 Timer | Clock generation (RUN mode) |
| 1 | SPDT Switch | LOAD/RUN mode selector |
| 2 | Pushbuttons | Manual WRITE and CLOCK control |
| 1 | 8-bit DIP Switch | Byte input during LOAD mode |
| 8 | LEDs | Output display for memory byte |
| 8 | 10kΩ Resistors | Pull-downs for DIP switch |
| 8 | 220–470Ω Resistors | LED current limiting |
| 1 | Breadboard | Prototyping platform |
| - | Jumper Wires | Wiring interconnects |
| 1 | 5V Power Supply | Logic and memory voltage source |

## Build Steps

### Step 1: Program Counter

- Chain 2× SN74LS590N counters for 16-bit address generation

- Connect 555 timer output (or manual step button) to CP0 of lower counter

- Connect RC of lower counter to CP0 of upper counter

- Output Q0–Q15 → connect to SRAM A0–A15

### Step 2: SRAM Wiring

- Tie SRAM /OE LOW to always enable output

- Connect /WE to a pushbutton (active LOW) for manual write

- Route D0–D7 to:

  - DIP switch during LOAD mode

  - LED bank during RUN mode via mode switch or jumper swaps

### Step 3: Manual Programming (LOAD Mode)

- Set mode switch to LOAD

- Use DIP switch to set 8-bit data value

- Press WRITE to store value into SRAM

- Press CLOCK to increment address counter

- Repeat steps 2–4 for each byte of program

### Step 4: Program Execution (RUN Mode)

- Set mode switch to RUN

- Start 555 timer (or step manually)

- Observe LEDs display stored data sequentially

## Completion Criteria

- SRAM can be loaded manually with 8-bit values

- Counter steps through addresses properly

- LEDs display expected data from memory

- Prepares system for Phase 2 (register and ALU integration)