# Dino v0.0.1 — CPU‑Free 8‑Bit Sequencer

## Objective

Design a manual, CPU-free 8-bit sequencer that:

- Generates a 16-bit address stream using chained SN74LS590N counters and a 555 timer clock.

- Loads program bytes into SRAM via an 8-bit DIP switch in LOAD mode.

- Uses a 74LS645 octal bus transceiver to arbitrate the shared data bus, preventing contention between DIP switch, SRAM, and LED display.

- Switches cleanly between LOAD and RUN modes via SPDT-controlled DIR and /OE signals.

- Reads and displays stored data sequentially on an LED bank in RUN mode.

## Bill of Materials

| Qty | Component |
| --- | --- |
| 2 | SN74LS590N 8‑bit binary counters |
| 1 | MCM60256AP‑10 32KB SRAM |
| 1 | 555 Timer (clock generation) |
| 1 | SPDT switch (LOAD/RUN mode) |
| 2 | Pushbuttons (WRITE, CLOCK) |
| 1 | 8‑bit DIP switch (LOAD mode data input) |
| 8 | LEDs (RUN mode output) |
| 8 | 10 kΩ resistors (pull‑down for DIP switch) |
| 8 | 220–470 Ω resistors (LED current limiting) |
| 1 1 1 1 1 | 74LS645N octal bus transceiver 10 kΩ Resistor (R1) 100 kΩ Resistor (R2) 1 µF (C1) 0.1 µF (C2) |
| — | Breadboard, jumper wires, 5 V power supply |

## Bus Architecture

| Device | Bus Role | Mode |
| --- | --- | --- |
| DIP Switch | Drives bus | LOAD |
| SRAM | Drives & reads | LOAD & RUN |
| LED Display | Reads from bus | RUN |

74LS645 Control Signals

| Mode | /OE | DIR | Function |
| --- | --- | --- | --- |
| LOAD | Low | High | DIP → SRAM write |
| RUN | Low | Low | SRAM → LED read |
| IDLE | High | Low | Bus tri‑stated |

## Design Considerations

- Power‑on Reset: Include a reset mechanism (pushbutton or RC network) on SN74LS590 counters to initialize the address to 0 on startup.

- Decoupling: Place 0.1 µF bypass capacitors at each VCC‑GND pin of the SRAM, 74LS645, and SN74LS590 devices.

- Control‑Line Biasing: Add pull‑ups or pull‑downs on 74LS645 DIR and /OE so the bus has a defined state if the SPDT switch is between positions.

- Default IDLE State: IDLE mode tri‑states the bus; the DIP switch will float (use pull‑downs) and LEDs remain off.

- Resistor Placement: All LED current‑limit resistors go on the anode side; ensure DIP switch inputs have pull‑downs.

- Timing Constraints: Keep the 555-timer frequency well below the 74LS645 maximum toggle rate (∼25 MHz) and within the SRAM’s access time. We will generate ~6.8Hz at around 52.7% Duty Cycle with the provided C1, R1 & R2

- Diagram Reference: Consider adding a simple block diagram showing 74LS645 side‑A to DIP, side‑B to SRAM/LED bus.

- Terminology: Use “IDLE” consistently for tri‑state rather than mixing “FLOAT”.

## Edge‑Case Checks

- Global Reset & SRAM /CE: Ensure SN74LS590 and 74LS645 power up in a known state (use RC reset or pushbutton) and SRAM’s /CE is tied low with outputs truly tri‑stating on /OE high.

- Debounce Write/Clock Buttons: Add RC or Schmitt‑trigger debouncing to prevent mechanical bounce causing extra writes or steps.

- Synchronous Timing: Verify 555 clock edges meet SN74LS590 setup/hold times; ensure duty‑cycle respects LS family timing even at low frequencies.

- Glitch Immunity on Mode Switch: Use a two‑pole switch or RC delay so /OE changes only after DIR settles, avoiding transient bus contention.

- Biasing Between Modes: Confirm DIP switch side inputs have proper pull‑downs so they don’t float when /OE is high.

- Power Supply Decoupling: Place 0.1 µF bypass caps at every IC’s VCC/GND pins and consider a 4.7 µF bulk cap on the 5 V rail.

- Mode‑by‑Mode Verification: Test LOAD and RUN separately – verify bus integrity in each and decide if the IDLE tri‑state is needed in practice.

## Build Steps

### 1. Program Counter

- Chain two SN74LS590N counters for a 16‑bit address.

- Connect the 555 timer to the lower counter’s CP0.

- Chain the lower’s ripple carry (RC) to the upper’s CP0.

- Route Q0–Q15 → SRAM A0–A15.

### 2. Bus Transceiver Integration

- Place the 74LS645 between DIP switch and shared data bus:

  - Side A ← DIP switch lines

  - Side B ← SRAM D0–D7 and LED bank

- Drive DIR: High in LOAD, Low in RUN

- Drive /OE: Low to enable, High to tri‑state (IDLE)

### 3. Mode Switch Logic

Use the SPDT switch to set:

- LOAD: Enable transceiver, DIR = High, SRAM write enabled

- RUN: Enable transceiver, DIR = Low, SRAM output enabled

- IDLE (optional): /OE = High for all bus devices

### 4. Manual Programming (LOAD Mode)

- Set mode to LOAD

- Set DIP switch to desired byte

- Press WRITE (SRAM /WE low) to store

- Press CLOCK to increment address

- Repeat for each byte

### 5. Program Execution (RUN Mode)

- Set mode to RUN

- Start the 555 timer (or step manually)

- LEDs display memory contents sequentially

## Completion Criteria

- No bus contention in LOAD or RUN modes

- DIP switch drives only in LOAD

- SRAM drives only in RUN

- LEDs read bus correctly

- Bus lines tri‑stated when idle