# 📟 Motorola Motor Control Board (MMCB) Canvas

## 🎯 Objective

Design and build a historically-grounded, motor control system using the Motorola 6808 CPU and the MC6840 programmable timer. The system receives two 8-bit digital speed values over a serial interface from a modern MCU and generates corresponding PWM outputs to control two DC motors via an H-bridge. The build emphasizes transparency, timing clarity, and eventual framing for archival or demonstration purposes.

## 🧩 Architecture Overview

### Central Elements

- CPU: Motorola 6808

- PWM Generator: Motorola MC6840 (3× 16-bit timers)

- Program ROM: 28C256 EEPROM

- Data RAM: 62256 SRAM

- SPI Bridge: Modern MCU (e.g., STM32 or ESP32) → 74HC595 → Parallel latch (e.g., 74LS374)

- Output Stage: TB6612FNG H-Bridge x2 (Left, Right Motor)

### Timing System

- External crystal oscillator for 6808 (e.g., 1 MHz)

- MC6840 configured for fixed-frequency PWM output

- Serial input latches triggered by GPIO from modern MCU

## 🧠 Data Flow

- Input

  - Modern MCU sends 2×8-bit values (Left, Right) over SPI to 74HC595.

  - Values latched via GPIO strobe into 74LS374.

  - 6808 reads from memory-mapped addresses (e.g., $4000 = Left, $4001 = Right).

- Processing

  - 6808 applies optional deadband and clamping.

  - Maps processed values to duty cycle register values.

- Output

  - 6808 writes to MC6840 control and count registers.

  - MC6840 outputs PWM waveforms.

  - PWM signals fed to TB6612FNG to drive motors.

## 🛠️ Hardware Blocks

| Function | Component(s) | Notes |
| --- | --- | --- |
| CPU | Motorola 6808 | Central control |
| Timer / PWM | Motorola MC6840 | Memory-mapped timer module |
| Program Storage | 28C256 EEPROM | Firmware |
| RAM | 62256 SRAM | Variable + stack |
| Serial Latch Input | 74HC595 + 74LS374 | SPI bridge from modern MCU |
| Address Decode | 74LS138, 74LS00 | ROM/RAM/I/O region mapping |
| H-Bridge Output | TB6612FNG (x2) | Motor driver |
| Clock | Crystal Oscillator | For 6808 and MC6840 timing |
| Display (Optional) | LED or 7-segment | Debug or status output |

## 🧪 Development Phases

### Phase 1: Core CPU Bring-Up

- Confirm power, reset, clock

- Build ROM/RAM map

- Run LED blinky from ROM

### Phase 2: Timer Setup

- Address decode and write-enable MC6840

- Initialize PWM channels manually

- Scope PWM outputs

### Phase 3: SPI Data Interface

- Connect modern MCU to 74HC595

- Latch data into 74LS374

- Read from $4000/$4001 and verify with LED/logic analyzer

### Phase 4: Motor Control Loop

- Map values to PWM duty

- Drive H-bridge

- Tune for deadband/clamp region

### Phase 5: Display and Frame

- Add debug indicators

- Finalize perfboard layout for visibility

- Frame under acrylic with labeling

## 🧰 Tooling & Workflow

- Assembler: asxxxx or Easy68k

- Programmer: TL866II+ or equivalent

- Debugging: LEDs, logic analyzer, oscilloscope

- Framing: Acrylic panel, standoffs, wood or aluminum frame

## 📌 Expansion Ideas

- UART output for debugging

- Analog input with external ADC

- 3rd motor or auxiliary PWM output

- Direction control via GPIO

## 🔚 Completion Criteria

- 6808 reads and processes SPI-latched values

- MC6840 emits valid PWM waveforms

- Motors respond proportionally

- Entire system visible and documented for display