# Dino v0.1.0 — Build Instructions and Bill of Materials

## Objective

Upgrade Dino v0.01 from a simple logic sequencer to a functional 8-bit CPU core capable of performing arithmetic operations and register-controlled logic execution.

### Bill of Materials

| Qty | Part | Description |
| --- | --- | --- |
| 1 1 | Dino v0.0.1 74F382N | 8-bit Logic Sequencer 8-bit ALU (2x 4-bit sections) |
| 2 | SN74LS273N | 8-bit D-type registers (Register A and B) |
| 1 | DM74LS374N | Output latch for ALU result |
| 8 | LEDs | Output display for ALU result |
| 8 1 | 220–470Ω Resistors 74LS74 | LED current limiting for ALU result display D flip-flops for flag capture (Zero/Carry) |

## Build Steps

### Step 1: Instruction Sequencer Setup (Dino v0.01)

- Chain 2× SN74LS590N to form 16-bit Program Counter

- Connect 555 timer (or manual clock button) to CP0 of lower counter

- Connect RC of lower counter to CP0 of upper counter

- Route Q0–Q15 from counters to A0–A15 on SRAM

- Tie /OE on SRAM LOW, /WE connected to WRITE button (active LOW)

- Use DIP switch to set D0–D7 during LOAD mode

- In RUN mode, disconnect DIP switch and connect D0–D7 to LED bank

### Step 2: Memory Loading

- Set mode switch to LOAD

- Set DIP switch to desired byte

- Press WRITE to store byte to current address

- Press CLOCK to advance to next address

- Repeat until program is fully entered

### Step 3: Output Verification (Dino v0.01)

- Set mode switch to RUN

- Press reset if needed to reset counters

- Start timer or manually step through memory

- Observe LEDs reflect stored bytes in sequence

### Step 4: Register Wiring (Dino v0.1)

- Connect SN74LS273N to SRAM data bus for parallel loading (Register A)

- Connect second SN74LS273N for Register B loading

- Use DIP switch to test loading or use memory bytes as inputs

- Wire clock lines to both registers for synchronized latching

### Step 5: ALU Integration

- Connect Register A and B outputs to ALU inputs

- Set ALU control lines manually or via instruction decoding (future step)

- Connect ALU output to DM74LS374 latch

- Connect output latch to LED display through resistors

### Step 6: Manual Instruction Simulation

- Manually load LOAD_A, LOAD_B, ADD, and OUT equivalent instructions into SRAM

- Step through execution and observe results

### Step 7: Define Instruction Set (Preview)

| Opcode | Mnemonic | Description |
| --- | --- | --- |
| 0x01 | LOAD_A xx | Load Register A with value |
| 0x02 | LOAD_B xx | Load Register B with value |
| 0x03 | ADD | A = A + B |
| 0x04 | STORE_A xx | Store A to memory[xx] |
| 0x05 | OUT | Display A on ALU result LED bank |

## Completion Criteria

- Successfully execute 3–8 byte programs

- Observe correct ALU operations

- Use LED output to verify result

- Ready to expand to instruction decoder and flag-controlled branching